MATHEMATICS FOR ROBOTICS

PROJECT

ProfessorName:-KUMAR UJJWAL

Student Name:- IMMANDI SRI LIKHITA

Roll.No:-SE25MROB009

Smart Room Temperature Control Using Fuzzy Logic Controller

Abstract

This project presents the design and implementation of a Smart Air Conditioner using a Fuzzy Logic Controller (FLC). The controller regulates room temperature by adjusting cooling power based on temperature error and change in temperature.

Objective

To design a fuzzy logic based temperature control system using MATLAB Fuzzy Logic Designer and Simulink.

Inputs and Output

Input 1: Temperature Error
Input 2: Change in Temperature
Output: Cooling Power

Membership Functions

Triangular membership functions (trimf) were used.

Temperature Error:
NL = [-10 -10 0]
ZE = [-5 0 5]
PL = [0 10 10]

Change in Temperature:
N = [-30 -30 0]
Z = [-5 0 5]
P = [0 30 30]

Cooling Power:
Low = [0 0 50]
Medium = [25 50 75]
High = [50 100 100]

Fuzzy Rules

1. IF Error is NL AND Change is N THEN Cooling is High
2. IF Error is NL AND Change is Z THEN Cooling is High
3. IF Error is NL AND Change is P THEN Cooling is Medium
4. IF Error is ZE AND Change is N THEN Cooling is Medium
5. IF Error is ZE AND Change is Z THEN Cooling is Low
6. IF Error is ZE AND Change is P THEN Cooling is Low
7. IF Error is PL AND Change is N THEN Cooling is Low
8. IF Error is PL AND Change is Z THEN Cooling is Low
9. IF Error is PL AND Change is P THEN Cooling is Low

Mathematical Model

Error Equation:
E = Tset - Troom

Change in Error:
ΔE = E(k) - E(k-1)

Transfer Function:
G(s) = 1 / (10s + 1)

Simulink Implementation

The Simulink model contains Step block, Sum blocks, Unit Delay, Mux, Fuzzy Logic Controller, Transfer Function, and Scope blocks.

Results

The response curve shows stable and smooth temperature regulation. The fuzzy controller successfully adjusts cooling power dynamically.

Conclusion

The fuzzy logic based smart AC system effectively controls room temperature without requiring an exact mathematical model. The system provides smooth and intelligent cooling control suitable for real-time applications.